3A, 9A GPIO

#include <reg51.h>

sbit LED1 = P1^0;

sbit LED2 = P1^1;

void delay()

{

unsigned int i,j;

for(i=0;i<200;i++)

for(j=0;j<1000;j++);

}

void main()

{

while(1)

{

LED1 = 1;

LED2 = 0;

delay();

LED1 = 0;

LED2 = 1;

delay();

}

}

4b, 6b 16 bit mul

#include <reg51.h>

void main()

{

while(1)

{

unsigned int num1, num2;

unsigned long int product;

num1 = 0x2222;

num2 = 0xBBBB;

product = (unsigned long int)num1 * num2;

P0 = product & 0xFF;

P1 = (product & 0xFF00) >> 8;

P2 = (product & 0xFF0000) >> 16;

P3 = (product & 0xFF000000) >> 24;

}

}

3B array 16 32

#include <reg51.h>

void main()

{

unsigned int i;

unsigned int array[5] = {0x1111, 0x2222, 0x8888, 0x4444, 0xABCD};

unsigned long int sum = 0;

for(i = 0; i < 5; i++)

{

sum = sum + array[i];

}

while(1);

}

2B,14B,16B Hello world

#include <reg51.h>

#include <stdio.h>

void main(void)

{

SCON = 0x50;

TMOD = 0x20;

TH1 = 0xFD;

TR1 = 1;

TI = 1;

while(1)

{

printf("Hello World ! \n");

}

}

1B    Hexa to dec

#include <reg51.h>

void main(void)

{

unsigned char hexa = 0xCF;

unsigned char hundreds, tens, units;

ACC = hexa;

B = 10;

ACC = ACC / B;

units = B;

B = 10;

ACC = ACC / B;

tens = B;

hundreds = ACC;

P0 = units;

P1 = tens;

P2 = hundreds;

while(1);

}