1a,10a .Led blinking

Code:-

ORG 0000H

up:SETB P2.0

ACALL DELAY

CLR P2.0

ACALL DELAY

SJMP UP

DELAY:MOV R4,#35

H1:MOV R3,#125

H2:DJNZ R3,H2

DJNZ R4,H1

RET

END

2a,11a,19a .Square wave

Code:-

ORG 0000H

UP: SETB P2.0

ACALL DELAY

CLR P2.0

ACALL DELAY

SJMP UP

DELAY: MOV R4,#35

H1:MOV R3,#255

H2:DJNZ R3,H2

DJNZ R4,H1

RET

END

4a,5b,10b,12a,13a,20a .Stepper motor

Code:-

ORG 0000H

START:

MOV P2,#09H

ACALL DELAY

MOV P2,#0CH

ACALL DELAY

MOV P2,#06H

ACALL DELAY

MOV P2,#03H

ACALL DELAY

SJMP START

DELAY:

MOV R4,#50

D1:

MOV R3,#255

D2:

DJNZ R3,D2

DJNZ R4,D1

RET

END

7a,14a.7-segment

Code:-

ORG 0000H

START:

MOV P2,#0C0H

ACALL DELAY

MOV P2,#0F9H

ACALL DELAY

MOV P2,#0A4H

ACALL DELAY

MOV P2,#0B0H

ACALL DELAY

MOV P2,#99H

ACALL DELAY

MOV P2,#92H

ACALL DELAY

MOV P2,#82H

ACALL DELAY

MOV P2,#0F8H

ACALL DELAY

MOV P2,#80H

ACALL DELAY

MOV P2,#90H

ACALL DELAY

SJMP START

DELAY:

MOV R5,#10

H1:

MOV R4,#180

H2:

MOV R3,#255

H3:

DJNZ R3,H3

DJNZ R4,H2

DJNZ R5,H1

RET

END

13b,15b. Arduino based manual count

int x0,x1,x2,x3,x4,x5,x6,x7,x8,x9;

int delay_time = 200;

void setup()

{

pinMode(12, INPUT_PULLUP);

pinMode(1, OUTPUT);

pinMode(2, OUTPUT);

pinMode(3, OUTPUT);

pinMode(4, OUTPUT);

pinMode(5, OUTPUT);

pinMode(6, OUTPUT);

pinMode(7, OUTPUT);

}

void loop()

{

int sensorVal = digitalRead(12);

if(sensorVal == LOW)

{

x0 = true;

}

while(x0)

{

zero();

sensorVal = digitalRead(12);

if(sensorVal == LOW)

{

x1 = true;

x0 = false;

}

}

while(x1)

{

one();

sensorVal = digitalRead(12);

if(sensorVal == LOW)

{

x2 = true;

x1 = false;

}

}

while(x2)

{

two();

sensorVal = digitalRead(12);

if(sensorVal == LOW)

{

x3 = true;

x2 = false;

}

}

while(x3)

{

three();

sensorVal = digitalRead(12);

if(sensorVal == LOW)

{

x4 = true;

x3 = false;

}

}

while(x4)

{

four();

sensorVal = digitalRead(12);

if(sensorVal == LOW)

{

x5 = true;

x4 = false;

}

}

while(x5)

{

five();

sensorVal = digitalRead(12);

if(sensorVal == LOW)

{

x6 = true;

x5 = false;

}

}

while(x6)

{

six();

sensorVal = digitalRead(12);

if(sensorVal == LOW)

{

x7 = true;

x6 = false;

}

}

while(x7)

{

seven();

sensorVal = digitalRead(12);

if(sensorVal == LOW)

{

x8 = true;

x7 = false;

}

}

while(x8)

{

eight();

sensorVal = digitalRead(12);

if(sensorVal == LOW)

{

x9 = true;

x8 = false;

}

}

while(x9)

{

nine();

sensorVal = digitalRead(12);

if(sensorVal == LOW)

{

x0 = true;

x9 = false;

}

}

}

12b,18b.LCD Ardiuno

Code:-

#include <LiquidCrystal.h>

LiquidCrystal lcd(12, 11, 5, 4, 3, 2);

void setup()

{

lcd.begin(16, 2);

lcd.setCursor(0, 0);

lcd.print("Hello Arduino");

lcd.setCursor(0, 1);

lcd.print("Proteus LCD");

}

void loop()

{

}

11b,17b.serialArdiuno

Code:-

void setup()

{

Serial.begin(9600); // send and receive at 9600 baud

}

int number = 0;

void loop()

{

Serial.print("Number is ");

Serial.println(number); // print the number

delay(500); // delay half second between numbers

number++; // to the next number

}